Build a SOHO Network:

Tasks :

1.

Draw a network diagram for a small office/home office (SOHO) setup that includes: one broadband internet connection, one Wi-Fi router, two laptops, one smart TV, and one printer. Clearly label each device and connection type (wired or wireless).

ANS :

2.

List all the hardware components (with brand/model suggestions) you would need to physically build the SOHO network diagrammed above, including cables and any switches or extenders if needed.<br><br><em><strong>Hint:</strong> Check current listings on Flipkart or Amazon for typical device models used in Indian homes/offices.</em>

ANS :

| Component | Quantity | Suggested Brand/Model | Purpose |
| --- | --- | --- | --- |
| Broadband Internet Connection | 1 | Airtel Xstream Fiber / JioFiber | Internet access |
| Modem/ONT | 1 | ISP Provided ONT/Modem | Connects to ISP |
| Wi-Fi Router | 1 | TP-Link Archer C6 AC1200 | Provides wired and wireless networking |
| Wi-Fi Router | 1 | TP-Link Archer C6 AC1200 | Provides wired and wireless networking |
| Laptop 1 | 1 | HP 15s / Dell Inspiron 15 | Wired workstation |
| Laptop 2 | 1 | HP 15s / Lenovo IdeaPad Slim 3 | HP 15s / Lenovo IdeaPad Slim 3 |
| Smart TV | 1 | Samsung 43" Smart TV | Wireless entertainment device |
| Printer | 1 | HP LaserJet MFP 136w | Network printer |
| Cat6 Ethernet Cable (Modem → Router) | 1 | D-Link Cat6 | Wired connection |
| Cat6 Ethernet Cable (Modem → Laptop 1) | 1 | D-Link Cat6 | Wired connection |
| Cat6 Ethernet Cable (Modem →Printer) | 1 | D-Link Cat6 | Wired connection |
| Power Adapter (Router) | 1 | Included with Router | Power supply |
| Power Strip Extension Board | 1 | Anchor by Panasonic | Power distribution |

3.

Write step-by-step instructions to configure the Wi-Fi router for your SOHO network, including setting up a unique SSID, enabling WPA2 security, and changing the default admin password.

ANS :

Step 1: Physical Connections

- Connect the modem to the router's WAN port using an Ethernet cable.

- Connect Laptop 1 to a LAN port on the router.

- Connect Printer to another LAN port on the router.

- Power on modem and router.

Step 2: Login to Router

- Open a browser on Laptop 1.

- Enter:

192.168.1.1

- Login using default credentials.

Example:

Username: admin

Password: admin

Step 3: Change Default Admin Password

Navigate to:

Administration → System Settings

Change:

Old Password: admin

New Password: SOHO_Admin@2026

Save settings.

Step 4: Configure Unique SSID

Navigate to:

Wireless Settings

Set:

SSID: SOHO_Office_Network

Save.

Step 5: Enable WPA2 Security

Navigate to:

Wireless Security

Configure:

Security Mode: WPA2-Personal

Encryption: AES

Password: SOHO@2026Secure

Save.

Step 6: Configure DHCP

Navigate to:

LAN Settings → DHCP

Enable:

DHCP Server: Enabled
Start IP: 192.168.1.100
End IP: 192.168.1.200

Save.

Step 7: Connect Devices

Wireless Devices

- Laptop 2

- Smart TV

Connect to:

SSID: SOHO_Office_Network

Password: SOHO@2026Secure

Wired Devices

- Laptop 1

- Printer

Already connected through Ethernet.

4.

Configure IP addresses for all devices in your SOHO network: assign static IPs to the printer and smart TV, and use DHCP for the laptops. Show a table listing each device, its MAC address (make one up), and the assigned IP address.

ANS :

Network Information :

| Parameter | Value |
| --- | --- |
| Network Address | 192.168.1.0/24 |
| Subnet Mask | 255.255.255.0 |
| Default Gateway | 192.168.1.1 |
| DNS Server | 8.8.8.8 |

Device IP Address Table :

| Device | Connection Type | MAC Address (Example) | IP Addres | Assignment |
| --- | --- | --- | --- | --- |
| Wi-Fi Router | Wired/Wireless | 00:AA:11:BB:22:01 | 192.168.1.1 | Static |
| Laptop 1 | Wired | 00:AA:11:BB:22:10 | 192.168.1.100 | DHCP |
| Laptop 2 | Wireless | 00:AA:11:BB:22:20 | 192.168.1.101 | DHCP |
| Smart TV | Wireless | 00:AA:11:BB:22:30 | 192.168.1.20 | Static |
| Printer | Wired | 00:AA:11:BB:22:40 | 192.168.1.10 | Static |

Static IP Configuration

Printer

IP Address: 192.168.1.10
Subnet Mask: 255.255.255.0
Gateway: 192.168.1.1
DNS: 8.8.8.8

Smart TV

IP Address: 192.168.1.20
Subnet Mask: 255.255.255.0
Gateway: 192.168.1.1
DNS: 8.8.8.8

5.

Explain how you would troubleshoot if one of the laptops cannot connect to the internet in your SOHO network. List at least 3 possible causes and what steps you would take to diagnose and fix each issue.

ANS :

Cause 1: Wrong Wi-Fi Password

Symptoms

- Laptop sees Wi-Fi network.

- Connection fails.

Diagnosis

- Verify SSID:

SOHO_Office_Network

- Check entered password.

Fix

- Forget Wi-Fi network.

- Reconnect.

- Enter:

SOHO@2026Secure

Cause 2: DHCP Not Assigning IP Address

Symptoms

- Connected to Wi-Fi.

- No Internet access.

Diagnosis

Open Command Prompt:

ipconfig

Check IP address.

If:

169.254.x.x

DHCP failed.

Fix

Run:

ipconfig /release
ipconfig /renew

If still unsuccessful:

- Restart router.

- Verify DHCP server is enabled.

Cause 3: Router Not Connected to Internet

Symptoms

- Laptop connected to Wi-Fi.

- Websites do not open.

Diagnosis

Check:

- Internet LED on modem.

- Internet LED on router.

Try internet on Smart TV.

Fix

- Restart modem.

- Restart router.

- Check Ethernet cable between modem and router.

- Contact ISP if outage continues.

Cause 4: Wireless Adapter Disabled

Symptoms

- No Wi-Fi networks visible.

Diagnosis

Open:

Control Panel → Network Connections

Check Wi-Fi adapter status.

Fix

- Enable Wi-Fi adapter.

- Update wireless drivers.

- Restart laptop.

Cause 5: DNS Server Problem

Symptoms

- Google does not open.

- Wi-Fi connected.

Diagnosis

Run:

ping 8.8.8.8

If ping works but websites don't load, DNS is the issue.

Fix

Configure:

Preferred DNS: 8.8.8.8
Alternate DNS: 8.8.4.4