DAY : 1-5

1.

Assign static IP addresses to five devices (laptops or desktops) on your home or lab network, ensuring each is in the same subnet, and verify connectivity between them using the ping command.

ANS :

Network Configuration :

| Device | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- |
| PC1 | 192.168.1.10 | 255.255.255.0 | 192.168.1.1 |
| PC2 | 192.168.1.11 | 255.255.255.0 | 192.168.1.1 |
| PC3 | 192.168.1.12 | 255.255.255.0 | 192.168.1.1 |
| PC4 | 192.168.1.13 | 255.255.255.0 | 192.168.1.1 |
| PC5 | 192.168.1.14 | 255.255.255.0 | 192.168.1.1 |

All devices are in the same subnet (192.168.1.0/24).

Connectivity Test

Command:

ping 192.168.1.11

Example output:

Reply from 192.168.1.11: bytes=32 time<1ms TTL=128
Reply from 192.168.1.11: bytes=32 time<1ms TTL=128

Result: All five devices successfully pinged each other, confirming network connectivity.

2.

Set up a DHCP server (using your router or software like Packet Tracer/GNS3/VirtualBox) to automatically assign IP addresses to at least three devices, and confirm that each device receives a unique IP from the correct range.

ANS :

DHCP Server Settings

| Parameter | Value |
| --- | --- |
| Network | 192.168.1.0/24 |
| Start IP | 192.168.1.100 |
| End IP | 192.168.1.150 |
| Default Gateway | 192.168.1.1 |
| DNS Server | 8.8.8.8 |

IP Addresses Assigned Automatically

| Device | Assigned IP |
| --- | --- |
| Laptop1 | 192.168.1.100 |
| Laptop2 | 192.168.1.101 |
| Laptop3 | 192.168.1.102 |

Verification command:

ipconfig

Result: Each device received a unique IP address from the DHCP range.

3.

Configure a DNS service (using your router’s DNS settings or a tool like dnsmasq) so that typing a custom hostname (e.g., musicplayer.local) in a browser on any connected device opens a specific local web page or device interface.<br><br><em><strong>Hint:</strong> You can map hostnames to local IPs using the DNS service or by editing the hosts file for a quick test.</em>

ANS :

Goal

Typing:

musicplayer.local

should open a local web page.

Configuration

| Hostname | IP Address |
| --- | --- |
| musicplayer.local | 192.168.1.50 |

Method 1: Using DNS Server

Add an A Record:

musicplayer.local → 192.168.1.50

Method 2: Using Hosts File

Edit:

C:\Windows\System32\drivers\etc\hosts

Add:

192.168.1.50 musicplayer.local

Test

Open browser:

http://musicplayer.local

Result: The browser opens the local web page hosted at 192.168.1.50.

4.

Create two VLANs (e.g., VLAN 10: 'Workstations', VLAN 20: 'Smart Devices') on your switch or simulation tool, assign at least two devices to each VLAN, and demonstrate that devices in the same VLAN can communicate, but not with devices in the other VLAN.<br><br><em><strong>Hint:</strong> Use ping or file sharing to test connectivity between devices in different VLANs.</em>

ANS :

VLAN Configuration

VLAN 10 – Workstations:

| Device | IP Address |
| --- | --- |
| PC1 | 192.168.10.10 |
| PC2 | 192.168.10.11 |

VLAN 20 – Smart Devices

| Device | IP Address |
| --- | --- |
| PC3 | 192.168.20.10 |
| PC4 | 192.168.20.11 |

Testing Communication

Same VLAN

PC1> ping 192.168.10.11
Reply from 192.168.10.11

PC3> ping 192.168.20.11
Reply from 192.168.20.11

Different VLAN

PC1> ping 192.168.20.10
Request timed out.

PC4> ping 192.168.10.10
Request timed out.

Result: Devices within the same VLAN communicate successfully, while communication between different VLANs is blocked because inter-VLAN routing is not configured.

5.

Simulate internet access for your LAN by configuring a router with NAT (Network Address Translation), then test that devices on your LAN can access an external site (e.g., www.wikipedia.org) or a simulated external server.

ANS :

Router Configuration

Inside Interface (LAN)

192.168.1.1/24

Outside Interface (Internet)

203.0.113.10

NAT Configuration (Cisco Example)

Router(config)# access-list 1 permit 192.168.1.0 0.0.0.255
Router(config)# ip nat inside source list 1 interface GigabitEthernet0/1 overload
Router(config)# interface GigabitEthernet0/0
Router(config-if)# ip nat inside
Router(config)# interface GigabitEthernet0/1
Router(config-if)# ip nat outside

Connectivity Test

ping www.wikipedia.org

or

ping 8.8.8.8

Example output:

Reply from 8.8.8.8: bytes=32 time=25ms TTL=118

Result: Devices on the LAN can access external websites through NAT.